ASSIGNMENT 2

Section 1: Multiple Choice

1. Which of the following precautions should be taken before working on computer hardware?

a) Ensure the computer is plugged in to prevent electrostatic discharge.

b) Wear an anti-static wrist strap to prevent damage from electrostatic discharge.

c) Work on carpeted surfaces to prevent slipping.

d) Use magnetic tools to handle components more easily.

ANS: b) Wear an anti-static wrist strap to prevent damage from electrostatic discharge.

2. What is the purpose of thermal paste during CPU installation?

a) To insulate the CPU from heat.

b) To provide mechanical support for the CPU.

c) To improve thermal conductivity between the CPU and the heat sink.

d) To prevent the CPU from overheating

ANS: c) To improve thermal conductivity between the CPU and the heat sink.

3. Which tool is used to measure the output voltage of a power supply unit (PSU)?

a) Multimeter

b) Screwdriver

c) Pliers

d) Hex key

ANS: a) Multimete

4. Which component is responsible for storing BIOS settings, such as date and time, even when the computer is powered off?

a) CMOS battery

b) CPU

c) RAM

d) Hard drive

ANS: a) CMOS battery

Section 2: True or False

5. True or False: When installing a new hard drive, it is essential to format it before use.

ANS: TRUE

6. True or False: A POST (Power-On Self-Test) error indicates a problem with the CPU.

ANS: False – POST checks many parts not only the CPU.

7. True or False: It is safe to remove a USB flash drive from a computer without ejecting it first.

ANS: False – Removing without eject may corrupt or damage files.

Section 3: Short Answer

8. Describe the steps involved in installing a new graphics card in a desktop computer.

ANS:

- Turn off the computer and unplug the power cable.

- Open the case by removing the side panel.

- Find the PCIe x16 slot (the long slot on the motherboard for graphics cards).

- Remove the slot cover from the back of the case where the card will fit.

- Insert the graphics card firmly into the PCIe slot.

- Secure it with a screw so it doesn’t move.

- Connect power cables from the PSU if the card needs extra power.

- Close the case and plug the power cable back in.

- Turn on the PC and install/update the graphics drivers.

9. What is RAID, and what are some common RAID configurations?

ANS:

RAID stands for “Redundant Array of Independent/Inexpensive Disks.”

- It is a method of using multiple hard drives together to increase performance, protect data, or both.

RAID Configurations

- RAID 0 (Striping):

- Divides data evenly across two or more drives for faster read/write speeds.

- If any drive fails, all data is lost.

- Requires at least 2 drives.

- RAID 1 (Mirroring):

- Creates an exact copy of data on two drives.

- If one drive fails, data is still safe on the other drive.

- Requires at least 2 drives.

- RAID 5 (Striping with Parity):

- Distributes data and parity across 3 or more drives.

- Provides a balance of speed and data safety.

- Requires at least 3 drives.

- RAID 6 (Double Parity):

- Similar to RAID 5 but can handle failure of 2 drives at the same time.

- Provides extra safety for important data.

- Requires at least 4 drives.

- RAID 10 (1+0):

- Combines RAID 1 (mirroring) and RAID 0 (striping).

- Uses more storage space because of mirroring.

- Requires at least 4 drives.

Section 4: Practical Application

10. Demonstrate how to replace a CPU fan in a desktop computer.

ANS:

- Turn off the computer and unplug the power cable.

- Open the case by removing the side panel.

- Disconnect the old fan’s power cable from the motherboard.

- Unscrew or unclip the old fan/heat sink carefully and lift it out.

- Clean the CPU surface with a soft cloth or alcohol wipe to remove old thermal paste.

- Apply a small drop of fresh thermal paste on the CPU.

- Place the new CPU fan/heat sink on top of the CPU and align it properly.

- Secure it with screws or clips tightly.

- Reconnect the fan cable to the CPU_FAN port on the motherboard.

- Close the case and power on the PC to check if the fan is spinning.

Section 5: Essay

11. Discuss the importance of regular maintenance for computer hardware and provide examples of maintenance tasks.

ANS:

Regular maintenance of computer hardware is very important to keep computers running smoothly, prevent unexpected failures, and extend the life of the components. Just like we service cars to keep them working properly, computers also need regular care. Without maintenance, dust, heat, and wear can cause hardware to slow down, overheat, or even break. This can lead to loss of important files, slower performance, and extra repair costs.

One key maintenance task is cleaning the computer. Dust and dirt can accumulate inside the case, on fans, and on vents, blocking airflow and causing the system to overheat. Using a soft brush or compressed air to clean the inside of the computer helps prevent overheating and keeps it running efficiently. Another important task is checking and maintaining cooling systems, like CPU and GPU fans. Ensuring that fans are working properly and reapplying thermal paste on the CPU every few years can improve cooling performance.

Storage maintenance is also crucial. For hard drives, running disk cleanup, defragmentation (for HDDs), and checking for errors ensures data is stored and accessed efficiently. For SSDs, updating firmware improves speed and reliability. Additionally, updating drivers and BIOS keeps hardware compatible with new software and improves performance.

Other maintenance tasks include inspecting cables, checking for loose connections, and monitoring the computer for unusual noises or performance issues. Regular maintenance prevents hardware failure, reduces repair costs, and ensures that the computer works fast, safely, and reliably for a long time.