Assignment module 3: Understanding and Maintenance of Networks

Section 1: Multiple Choice

1. What is the primary function of a router in a computer network?

a) Assigning IP addresses to devices

b) Providing wireless connectivity to devices

c) Forwarding data packets between networks

d) Managing user authentication and access control

ANS: c) Forwarding data packets between networks

2. What is the purpose of DNS (Domain Name System) in a computer network

a) Encrypting data transmissions for security

b) Assigning IP addresses to devices dynamically

c) Converting domain names to IP addresses

d) Routing data packets between network segments

ANS: c) Converting domain names to IP addresses

3. What type of network topology uses a centralized hub or switch to connect all devices?

a) Star

b) Bus

c) Ring

d) Mesh

ANS: a) Star

4. Which network protocol is commonly used for securely accessing and transferring files over a network?

a) HTTP

b) FTP

c) SMTP

d) POP3

ANS: b) FTP

Section 2: True or False

5. True or False: A firewall is a hardware or software-based security system that monitors and controls incoming and outgoing network traffic based on predetermined security rules.

ANS: TRUE

6. True or False: DHCP (Dynamic Host Configuration Protocol) assigns static IP addresses to network devices automatically.

ANS: FALSE

REASON: DHCP gives dynamic IP addresses, not static ones.

7. True or False: VLANs (Virtual Local Area Networks) enable network segmentation by dividing a single physical network into multiple logical networks.
ANS: TRUE

Section 3: Short Answer

8. Explain the difference between a hub and a switch in a computer network.

ANS:

| Feature | Hub | Switch |
| --- | --- | --- |
| Function | Sends data to all devices | Sends data only to the correct device |
| Efficiency | Less efficient; causes more traffic | More efficient; reduces network traffic |
| Intelligence | No intelligence; works at Layer 1 | Intelligent; works at Layer 2 |
| Speed | Slower | Faster |
| Security | Low (data goes to everyone) | High (data goes to the intended device only) |

9. Describe the process of troubleshooting network connectivity issues.

ANS:

- First of all, check if all cables are plugged in properly and the router has powered.

- Make sure Wi-Fi is turned on in your device.

- Try to open a website on another device to see if the internet works or not.

- Restart your router and your device.

- Check if your device is getting a proper IP address.

- Try pinging the router to see if your device can reach it.

- If the router works but internet doesn’t, try changing the DNS or pinging an external IP.

- If still not works, the problem may be with your ISP call them for help.

Section 4: Practical Application

10. Demonstrate how to configure a wireless router's security settings to enhance network security.

ANS:

- Log in to the router
Connect your device to the router, open a web browser, enter the router’s IP address (such as 192.168.0.1), and log in using the admin details.

- Rename the wireless network (SSID)
Change the default Wi-Fi name to something unique that does not reveal personal information or the router brand.

- Enable strong encryption
Go to the wireless security settings and select WPA3 or WPA2-PSK (AES) to protect data on your network.

- Create a strong Wi-Fi password
Set a long password using uppercase letters, lowercase letters, numbers, and symbols.

- Disable WPS
Turn off Wi-Fi Protected Setup to prevent unauthorized access through security loopholes.

- Change administrator login credentials
Update the router’s admin username and password so only authorized users can modify the settings.

- Enable the firewall
Switch on the router’s built-in firewall to block suspicious or unwanted traffic.

- Save and apply settings
Save all changes and reboot the router to ensure the new security settings take effect.

Section 5: Essay

11. Discuss the importance of network documentation and provide examples of information that should be documented.

ANS:

Importance of Network Documentation

Network documentation is essential for the effective management, maintenance, and security of a computer network. It provides a clear record of how the network is designed, configured, and operated. Proper documentation helps network administrators understand the network quickly and reduces errors during troubleshooting or upgrades.

Key importance includes:

- Troubleshooting: Makes it easier to identify and fix network problems quickly.

- Maintenance and upgrades: Helps technicians perform updates or expansions without disrupting services.

- Security: Supports better security management by recording access controls, device settings, and policies.

- Disaster recovery: Assists in restoring the network after failures or cyberattacks.

- Knowledge sharing: Ensures continuity when staff change or new technicians join.

Examples of Information to Document

- Network topology

  - Network diagrams (physical and logical)

  - Device connections and layout

- IP addressing information

  - IP address ranges

  - Subnet masks

  - Default gateways

  - DHCP settings

- Network devices

  - Routers, switches, firewalls, and access points

  - Device models, serial numbers, and locations

- Configuration settings

  - Router and switch configurations

  - VLAN settings

  - Wireless SSIDs and security modes

- Security details

  - Firewall rules

  - Access control lists (ACLs)

  - Authentication methods and policies

- User and access information

  - Network user roles

  - Permissions and access levels

- Maintenance records

  - Firmware updates

  - Configuration changes

  - Troubleshooting logs